# FDEM_2D User’s Manual

Written by Scott Stapleton

# Input Files

General format: program is run via text input files.  Output can be requested in the form of .csv files to be read by PlotFDEM.

## Sample Input File:

** Fiber DEM Analysis Input File

** Scott Stapleton

**

**________________________________________________________________

**

*ScaleFactors

**

Escale	1.0		Scale Factor for the fiber modulus

Mscale	1.0		Scale Factor for the fiber mass

**

**________________________________________________________________

**

*Fiber

**

E1	240000		E axial (MPa)

E2	15000		E transv(MPa)

nu	0.27		Poissons ratio (not really continuum mech kosher)

r	3.5		radius of fiber (um)

l	35.0		length of fibers (um)

linDens	6.811E-8	linear density of a fiber (mg/um)

dGlobal	1.0		global damping (1=critical damping)

**

**________________________________________________________________

**

*Contact

**

dCOF	0.35		dynamic coefficient of friction

sCOF	0.35		static coefficient of friction

dCon	0.0		contact damping factor (1=critical damping)

Kn/Kt	1.33		normal stiffness over tangent stiffness for contact

**

**________________________________________________________________

**

*Matrix

**

EMat	4000.0		matrix stiffness (MPa)

nuMat	0.4		matrix poisson's ratio

SigMax	1500.0		Maximum Stress when the matrix breaks (MPa)

dMat	1.0		damping coefficient factor (1-critical damping)

nAnal	1		analysis number to start adding matrix

charDis	0.05

Model	MatrixContinuumElasticFibers

** Models: MatrixContinuum, MatrixContinuumElasticFibers

**

**________________________________________________________________

**

*RVE

**

Vf	0.50		volume fraction of fibers to empty space

pType	Random		Random, Hex

nRows	7		number of rows (total fibers is nRows^2)

nRep	1		number of repetitions of the RVE

MinSpacing	0.2

ShowResults	True

SaveResults	True

UndampedSteps	300

MaxSteps	40000

BoundaryType	p/s	y and z boundaries:  p=periodic, s=solid

** Options for Random: MinSpacing, FPerSquare, Margin, ShowResults,

** SaveResults, ConDamp, GlobDamp, IncrDamp, MaxSteps, UndampedSteps, BoundaryType

**________________________________________________________________

**

*Analysis

**

nAnaly	1		number of different load cycles

** For each step:

** Analysis Type: QuasiStaticAnalysis, ConstantVolFractionLoadAnalysis, RelaxationStepAnalysis, LoadStepAnalysis

** Constants needed

** change in strain for each cycle: 11, 22, 33, 23, 13, 12

** QuasiStaticAnalysis: nLoadSteps, nRelaxSteps, nLoadingSteps, perKE

** LoadStepAnalysis: nTimeSteps, nStepsToSkip, nStepsToSkipContact

** LoadStepAnalysis: nTimeSteps, nStepsToSkip, nStepsToSkipContact

** RelaxationStepAnalysis: maxSteps, nStepsPerRecording, nSwithoutDamp, DampCoeff, %KETot

** ConstantVolFractionLoadAnalysis:  nTimeSteps, nStepsToSkip, nStepsToSkipContact, totS, sComp, Vratio

** ConstantVolFractionQuasiStaticAnalysis:  nLS, nLSperRec, nRelaxS, nLingS, dampCoeff, %KE, totS, sComp, Vratio

**

**

**aType1	LoadStepAnalysis

**cons	1000, 100, 1		 nLoadSteps, nRelaxSteps, nLoadingSteps, dampingCoeff, perKE

**dS	0.0, 0.01, 0.0, 0.0, 0.0, 0.0

**

aType1	QuasiStaticAnalysis

cons	100, 1, 100, 1000, 0.001		 nLoadSteps, nRelaxSteps, nLoadingSteps, **dampingCoeff, perKE

dS	0.0, 0.001, 0.0, 0.0, 0.0, 0.0

**

**aType1	QuasiStaticAnalysis

**cons	100, 1, 100, 100, 0.001		 nLoadSteps, nRelaxSteps, nLoadingSteps, dampingCoeff, perKE

**dS	0.0, -0.001, 0.0, 0.0, 0.0, 0.0

**

**aType1	LoadStepAnalysis

**cons	1000, 10, 1		 nLoadSteps, nRelaxSteps, nLoadingSteps, dampingCoeff, perKE

**dS	0.0, 0.0, 0.0, 0.0, 0.0, 0.0

**_______________________________________________________________

**

*Output

**

bAll	True

b1st	False		Only output everything on the first specimen

bSS	False		Outputs the stress vs Strain

bPlotKE	True		Plot the kinetic energy of all time steps

**

**________________________________________________________________

**

*Experiment

**

expType	RVESize		Analysis Type (RVESize, NTimeSteps,RVESizeAndNTS,SizProb, SingleRun)

cons	10		constants separated with comma required for the analysis type

**

**________________________________________________________________

**

*END*END

## General Formatting Comments:

- ** = comment

- *= section title

- Use lines to separate modules (titles)

- Try to keep modules in the order given

- Must have *END

## Modules

### *ScaleFactors

You can define factors for scaling the modulus or mass.  I believe that both of these must be specified.  I leave this at 1.0 for now.

Escale 1.0

Mscale 1.0

- Type: Escale.

- Value:

  - 1.0 denotes no modulus scaling, > 1.0 would be positive modulus scaling.  This is often used for slow simulations to change the minimum time step and allow faster convergence.

- Type: Mscale.

- Value:

  - 1.0 denotes no mass scaling, > 1.0 would be positive mass scaling.  This is often used for slow simulations to change the minimum time step and allow faster convergence.

### *GRID

The Grid is the space in which contact is calculated.  It has an origin, size, and cell size.  It is recommended that the transverse cell size (y,z) is larger than the largest “fiber”.  If a fiber is larger than the cell size, it should be created as a large fiber, as this will give multiple points per contact plane.  Cell size is a minimum, will be rounded up to fit the grid in an even integer.

*Fiber

**

E1	240000		E axial (MPa)

E2	15000		E transv(MPa)

nu	0.27		Poissons ratio (not really continuum mech kosher)

r	3.5		radius of fiber (um)

l	35.0		length of fibers (um)

linDens	6.811E-8	linear density of a fiber (mg/um)

dGlobal	1.0		global damping (1=critical of contact damping)

- Type:

  - E1

    - Fiber stiffness axial (MPa) in the axial (x) direction.  Currently just used for the stress when a an x-direction strain is applied.

### *SolidObjectTypes

*Contact

**

dCOF	0.35		danymic coefficient of friction

sCOF	0.35		static coefficient of friction

dCon	0.0		contact damping factor (1=critical damping)

Kn/Kt	1.33		normal stiffness over tangent stiffness for contact

- Name: Any name for the function.  No spaces, cap sensitive.

- Type: (see sub-sections for explanation

  - dCOF

    - dynamic coefficient of friction between fibers

  - sCOF

    - static coefficient of friction between fibers

  - dCon

    - friction damping coefficient between fibers.  1.0 means critical damping, 0.0 is no damping.

  - Kn/Kt

    - Ratio of normal to shear contact stiffness.  Normal contact determined by material properties, this determines tangential stiffness.

#### Fiber

Fiber is an elastic, prismatic member that is smaller than the grid size.

Name, Fiber, E1, E2, nu, r, linDens, dAxial, dGlobal

- E1 is the axial modulus of the fiber in MPa

- E2 is the transverse modulus of the fiber, assuming transverse isotropy.  Also in MPa

- Nu is the Poisson’s ratio of the fiber.  Not really fit for a detailed continuum analysis, but gives contact properties based on E and nu

- r is the radius of the fiber in micrometers

- l is the length of the fibers in micrometers

- linDens is the linear density of a fiber, in mg per um

- dAxial is the axial damping coefficient for each fiber segment. 1.0 is critically damped, 0 is undamped, and anything over 1.0 is overdamped.

- dGlobal is the global damping coefficient for each fiber.  1.0 is critically damped, 0 is undamped, and anything over 1.0 is overdamped.  However, this damping is just considering the fiber mass and stiffness, so it isn’t really critical damping.  It is usually too much damping for large problems.

#### LargeFiber

A Large Fiber is an elastic, prismatic member that is larger than the grid size.  It has to have multiple contact points in one cross-section, so it is much more work to calculate and use, so use sparingly.

Name, LargeFiber, E1, E2, nu, r, linDens, dAxial, dGlobal

- Same as Fiber above

- Not yet implemented

#### LargeObject

Not sure yet what this will be.  It is not implemented, and not read.  It is a placeholder for an object that is not prismatic like a fiber, but it’s cross-section engulfs multiple cells.

Name, LargeObject, E1, E2, nu, linDens, dGlobal, Function

- Not Yet implemented

#### Wall

Placeholder for a straight wall that has a stiffness and can be contacted.

Name, Wall, E, nu, l, arealDens, dGlobal

- Not Yet implemented

### *NonSolidObjectTypes

A nonsolid object is something that does not necessarily have mass and induce contact, but still interacts with the solid objects in the simulation

**Name, Type, Constants

- Name: Any name for the function.  No spaces, cap sensitive.

- Type: (see sub-sections for explanation

  - Boundary

  - Liquid Large Object

  - Sizing

#### Boundary

A boundary does not have mass or cannot contact a solid object, but it can control it’s displacement or make it projected.

Name, Boundary, size x/y/z, BC Type x/y/z (Per, Disp, or None)

- x,y,z components of the size of the boundary.  The origin (location in instancing) is the exact center in all dimensions, so the walls of the boundary will be placed around it

- Boundary condition type in x/y/z direction. Can be:

  - Per : Periodic, or creates projected objects on the opposite boundary

  - Disp: this creates a displacement boundary condition for any object touching this boundary

  - None: pretty obvious☺

#### Liquid

This is a placeholder for something that effects the damping of an object when it comes into contact with it.

Name, Liquid, size x/y/z, viscosity

- x,y,z components of the size of the liquid (assuming rectangular, for filling a boundary).  The origin is the exact center in all dimensions.

- Viscosity of the liquid (units: TBD)

### *ObjectInstances

This is how you actually place objects into the space for the simulation.  I imagine that this section will change frequently as things develop.  All objects must be here, or they will not be added.

**Name, Object, Iteration, Type, Constants

- Name: Name of Instance(s).  Any name: no spaces, cap sensitive.

- Object: The name of the object that you are creating an instance of.  Cap sensitive.

- Type: the type of instance

  - Single: Creates a single instance of an object

    - Location (x/y/z): x, y, z coordinates of the instance.  How this is used depends on the object, as this is handled at the object level

    - Orientation (x/y/z): x, y, z components of the orientation.  How this is used depends on the object, as this is handled at the object level.

  - HexPacked: Hex-Packed in the YZ Plane, straight in X-dir

    - Box Location (x/y/z): x, y, z coordinates of the instance.  How this is used depends on the object, as this is handled at the object level

    - Box Size (x/y/z): x, y, z components of the orientation.  How this is used depends on the object, as this is handled at the object level.

    - Volume Fraction: ratio of the volume of the instances in the box and the box volume.  When the number cannot be reached exactly, it will be rounded down.  1.0 is only fibers, and 0 is no fibers.  Calculated simply as the area of a fiber * number of fibers divided by volume of the RVE.

  - Entangled: Randomly distributed instance locations in the XY Plane, entangled with normal function in the XY and XZ planes

    - Box Location (x/y/z): x, y, z coordinates of the instance.  How this is used depends on the object, as this is handled at the object level

    - Box Size (x/y/z): x, y, z components of the orientation.  How this is used depends on the object, as this is handled at the object level.

    - Volume Fraction: ratio of the volume of the instances in the box and the box volume.  When the number cannot be reached exactly, it will be rounded down.  1.0 is only fibers, and 0 is no fibers.  Calculated simply as the area of a fiber * number of fibers divided by volume of the RVE.

    - StDev (XY/XZ): Standard deviation of orientation normal distribution function for the XY and XZ planes in degrees (°) .

    - NDiv: The number of divisions of the prismatic fibers for the angle calculation.  For example, when this is set to 1, the fibers will be rotated so that they are still straight.

### *INteraction

**Type, Constant

There needs to be a contact for each solid object interaction, including itself.  Contact stiffness is dependent on the material properties of the objects involved in contact.

- Contact

- Solid Object 1: the name of one of the solid objects involved in the contact

- Solid Object 2: the name of one of the other solid object involved in the contact

- dCOF:  is the dynamic coefficient of friction for the contact

- sCOF:  is the static coefficient of friction for the contact

- dCon:  is the contact damping, based on two masses with a spring in-between.  1.0 is critical, 0.0 is no damping

- Kn/Kt:  is the ratio of the normal stiffness of the contact to the tangential stiffness.  I haven’t yet found a sound reason for doing this the way I did, so I chose 1.33.  INCLUDE CITATION

- Solid Object 1: the name of one of the solid objects involved in the contact

- Sizing (old, ignore)

  - E: modules

  - Nu: poisson ratio

  - Strength:

  - Prob: probability of having sizing

  - Dmax: max damping coeff

  - Iteration: iteration to apply sizing

### *BC

This defines any prescribed displacements or forces (or rotational equivalent).  Force/Moment boundary conditions are applied after the predictor step, while displacement/rotation boundary conditions are applied after the corrector step.

**Name, Object, Type, cons

- Name: Any name for the BC.  No spaces, cap sensitive.

- Object: Name of any object, solid or non-solid.  No spaces, cap sensitive

- Type:

  - Force or Disp: Prescribed force/displacement applied to an object

    - Magnitude: Magnitude of the force/displacement

    - Direction: Direction of the force/displacement (x, y, or z)

  - Moment or Rot: Prescribed moment/rotation applied to an object

    - Magnitude: Magnitude of the moment/rotation in the x-direction.  Positive and negative follow right-hand rule

  - Boundary: Prescribed strain to the boundary (Object must be of type boundary)

    - Strain (xx/yy/zz/xy/xz/yz): Applied strain to the boundary

  - ConsVBoundary: Prescribed strain to the boundary (Object must be of type boundary).  Constant volume is maintained during the straining.

    - StrainComponent: 2=Ezz, 3=Exy, 4=Exz, 5=Eyz.  The other strain components are defined to keep the volume constant.

    - StrainMag: Magnitude of the applied strain to the boundary

### *Analysis

The analysis is a collection of boundary conditions and objects.  There are many types, and they are executed in numerical order.  TODO: Make some way to erase strains between analysis steps someday.

** Number, ObjectInstances(sep by /), NonSolidObjects (sep by /), BCs (sep by /),

**		outputs (sep by /), Type, cons

- Number: Numbers are automatically assigned, so this is mainly for the user to have reference to the number when requesting output.  Count from 0 up.

- ObjectInstances:

  - All: all the object instances

  - Specific object instances separated by /

- NonSolidObjectTypes:

- BCs: This is a list of the names of boundary conditions, which will be applied during the loading.  Separate all BCs with a “/”.  All BCs will be divided out evenly over all iterations.

- Outputs: output name, that indicates what output

- Type:

  - LoadAnalysis: Load simply applied over a number of steps

    - nTimeSteps: Total number of time steps

    - nStepsToSkip:  number of steps to skip for recording output.  0 would be recording output at every iteration.

    - : Number of steps to skip without updating the grid.  For slow-moving simulations, this can be quite high.  0 would be re-assigning objects to the grid at every iteration.

## *Sizing

Defines the equivalent-area truss/spring model used to connect non-contacting fiber pairs ("sizing" springs), including its elastic properties and modular failure criterion. Sizing springs use the same IFailureCriteria/MaterialModel abstraction as *Matrix.

### dMaxSiz

Maximum initial center-to-center distance between two fibers for a sizing spring to be created between them.  In length scale

### ESiz

Elastic (Young's) modulus of the equivalent sizing material.

### NuSiz

Poisson's ratio of the equivalent sizing material.

### dSiz

Damping coefficient for the sizing spring (used to compute critical damping in the normal and tangential directions).  1=critically damped

### nAnalysSiz

Number of analysis steps to wait before evaluating/creating sizing springs.

### probSiz

Probability (0–1) that a sizing spring is created between an eligible fiber pair.

### failureCriteriaNameSiz

Name of the failure criterion used to evaluate spring failure. Must match one of the criteria supported by CreateFailureCriteria (e.g. VonMises, MaxPrincipalStress, MaxPrincipalStrain, FractureEnergyPrincipalStress, MaxPrincStressWithDamage, MaxPrincStressWithDamage2, MaxPrincStrainWithDamage, MaxPrincStressZIntDNoDamage, NoFailure). Failure is currently brittle: once the criterion is met, the spring breaks completely (progressive damage is not yet implemented for sizing).

### failureConstantsSiz

Slash-separated (/) list of numeric constants required by the chosen failureCriteriaNameSiz (e.g. a single critical stress value for VonMises). Leave blank when using NoFailure.

### *Experiment

This is used to automate parametric studies.  Use any experiment with 1 repetitions to run just one.  If multiple are defined, they will be run in the order that they appear.  If you don’t want any parametric study, then just put in here “SingleRun” and have no constants.

**Repetitions, Type, cons

- Name: Name of Instance(s).  Any name: no spaces, cap sensitive.

- Type:

Object: The name of the object that you are creating an instance

### *Output

This generates output.  The more output you suppress, the smaller your files will be and the faster the analysis.

**Type, cons

- Type:

  - State: gives an output file of the state of the objects, as many as are specified

    - Analysis / : the analysis numbers that will be given in the state file

    - 1st Only: true or false whether to write out the state of the first model only

    - Objects /: name of all the objects that could be concluded.  Use names defined earlier.  Other than object names, could include Contact,

  - History: saves a output file of some history variable for more compact output files

    - Analysis: the analysis numbers that will be given in the state file

    - BCs /: name of all the boundary conditions to be included.  Also included are the keywords “KE”.

  - Monitor: Displays output in real time in another thread so that one can monitor the progress of the simulation.

    - Analysis: the analysis numbers that will be given in the state file

    - BCs /: name of all the boundary conditions to be included.  Also included are the keywords “KE”.

## Running from the command LINE

FDEM_2D can also be run from a command line.  All you need to do is type in the location of the FDEM_2D.exe file, a space, and then optional input arguments separated by a space.  The optional input arguments can be file locations or folder locations for input files.  The program with run a specified input file, or search for and run every .txt file in a specified directory.  Output will be named according to input file names and other naming conventions.

# Programming Guidelines

Each class should be divided up into private properties, public, constructors, private methods, and public methods in this template:

#region Private Members

#endregion

#region Protected Members

#endregion

#region Public Members

#endregion

#region Constructors

#endregion

#region Private Methods

#endregion

#region Protected Methods

#endregion

#region Public Methods

#endregion

All variables should be private, with public get/set properties which call on the private properties when needed.

Private variables generally lowercase, while public are uppercase with the same name

# Running on Command Line

You can run FDEM from the commandline and pass in arguments.  The arguments can be a file name (with full or relative path) or a directory (with full or relative path).  If it’s a directory, it’ll try to run all .txt files in the directory.  You can separate directories with a space.  Like this:

“E:\Google Drive\IFAM\Projects\FDEM\Programs\FiberDEM_2D\fiberdem\bin\Debug\FiberDEM.exe” “E:\Google Drive\IFAM\Projects\FDEM\Programs\FiberDEM_2D\fiberdem\bin\Debug\Test\Dummy2” “E:\Google Drive\IFAM\Projects\FDEM\Programs\FiberDEM_2D\fiberdem\bin\Debug\Test\Dummy”

(I had to put the quotes around it because there is a space in Google Drive, and the computer doesn’t like it.)

I made it so that it starts each file on a separate thread up to the number of cores.